How to configure ISUP in Yate

We will set up an ISUP configuration in Yate to make calls.
Below is an image that describes the configuration.
So there are 2 Yate that are configured as a softswitch, each of them will act as a SIP server and a SS7 SEP in the network. The Clients used will be Yate SIP Clients that will communicate with Yate on SIP protocol, then the call is routed through SS7 to the corresponding SEP. The message will be read at each level in the order shown in the image: SCTP, M2PA, MTP3, ISUP and then routed in SIP and sent to the destination.

This is based on the modified yate from my repo:

https://github.com/svinson1121/yate/tree/main

** The STP in the above image is optional.

ISUP configuration Prerequisite

- SCTP:

On the machines where ISUP will be configured SCTP must be installed and compiled.

- Yate:

Download a copy of Yate

Yate needs to be configured with command: ./configure --enable-sctp=yes.

Parameters & Source

Parameters:

- pointcodetype

- format

- remotepointcode

- lockgroup

- earlyacm

- inn

- numplan

- numtype

- presentation

- screening

- callercategory

- channelsync

- userparttest

- t9

- continuity

- confirm_ccr

- drop_unknown

- ignore-grs-single

- ignore-cgb-single

- ignore-cgu-single

- duplicate-cgb

- parttestmsg

- ignore-unknown-digits

- sls

- maxcalleddigits

- print-messages

- extended-debug

Source:

- libs/ysig/isup.cpp

Building the setup

We are going to use 2 Softswitches with Yate installed and configured with ISUP to make calls (see the configuration for Server1 and Server2).
Each ISUP will have the same list of circuits.
The transport will be done over SCTP and is set in sigtransport.conf file.
The configuration of SS7 signaling will be done in ysigchan.conf and will consist of: Linkset: MTP3, Link: m2pa and Trunk: ISUP.
The voice is configured in mgcpca.conf.

For transmitting the voice another machine will be used (see Server3) that will act as a Media Gateway.

Server1 (IP:192.168.168.156)

Transport it is done over SCTP:

sigtransport.conf

;The name of the section identifies the connection

[sctp-m2pa]

type=sctp

stream=false

;Primary local address

local=192.168.168.156:5050

remote=192.168.168.200:6999

SS7 signalling configuration

ysigchan.conf

;SS7 MTP3 network (linkset)

[mtp3]

type=ss7-mtp3

;Comma separated list of point code types used to map an

;incoming network indicator to a specific point code type

netind2pctype=ITU

netindicator=national

;Declare a local pointcode for the SS7 network

local=ITU,0-0-6

;Build an adjacent route for the SS7 network

adjacent=ITU,0-0-5

;Name of a SS7 Layer 2 link to create in the linkset

link=m2pa

;SS7 M2PA link

[m2pa]

type=ss7-m2pa

sig=sctp-m2pa

;SS7 ISUP trunk

[isup]

type=ss7-isup

netindicator=national

pointcodetype=ITU

pointcode=0-0-6

remotepointcode=5

defaultpointcode=6

voice=mgcpca

Voice it is set in:

mgcpca.conf

[gw mgcpca]

user=ACgw100

media_token=yes

host=192.168.168.77

port=2427

address=192.168.168.77

voicechans=1-15.17-31

chans=31

start=1

[engine]

Configure user for test:

regfile.conf

[333]

password=mysecret1

Server2 (IP:192.168.168.200)

Transport it is done over SCTP:

sigtransport.conf

[sctp-m2pa]

type=sctp

stream=false

local=192.168.168.200:6999

remote=192.168.168.156:5050

SS7 signalling configuration:

ysigchan.conf

[mtp3]

type=ss7-mtp3

netind2pctype=ITU

netindicator=national

local=ITU,0-0-5

adjacent=ITU,0-0-6

link=m2pa

[m2pa]

type=ss7-m2pa

sig=sctp-m2pa

[isup]

type=ss7-isup

netindicator=national

pointcodetype=ITU

pointcode=0-0-5

remotepointcode=6

defaultpointcode=5

voice=mgcpca

Voice it is set in:

mgcpca.conf

[gw mgcpca]

user=Acgw200

media_token=yes

host=192.168.168.77

port=2427

address=192.168.168.77

voicechans=1-15.17-31

chans=31

start=1

[engine]

Configure user for test:

regfile.conf

[222]

password=mysecret2

regexroute.conf

[default]

.*=sig/\0;trunk=isup

Server3 (IP:192.168.168.77)

Configuring mgcp gateway and the 2 EndPoints that are set in each machine (in Server1 and in Server2):

mgcpgw.conf

[engine]

enabled=yes

address=0.0.0.0

request_ack=no

send_provisional=no

retrans_interval=0

retrans_count=0

[ep ACgw100]

local_user=ACgw100

local_host=192.168.168.77

remote_host=192.168.168.200

[ep ACgw101]

local_user=Acgw200

local_host=192.168.168.77

remote_host=192.168.168.156

Testing the setup

Server1 and Server2 started then Server3.
Two Yate clients can be used to register the users configured. Then calls are made from one client to another.